1. State maximum shear stress theory of failure. Where do you use maximum shear stress theory of failure?

According to the maximum shear stress theory, a material will fail or yield when its maximum shear stress equals or exceeds the shear stress value at the yield point in the uniaxial tensile test. This theory is suitable for ductile materials. The maximum shear stress theory was given by Henry Tresca. Hence Maximum shear stress theory is also known as Tresca's theory of failure.

The maximum shear stress theory is used for ductile materials. It is used for biaxial and triaxial loading.

2. Draw stress-strain diagram for mild-steel rod. Show and discuss its different limits on it.

Where, A = Limit of proportionality, B = Elastic limit, C = Upper yield point, D = Lower yield point, E = Ultimate load point, F = Breaking point

3. Illustrate how the stress concentration in a component can be reduced.

Some of the most common practical methods to reduce stress concentrations in machine parts are:

1. Provide gradual transitions between changes in cross-sections

2. Provide relief notches

3. Use several smaller relief notches instead of a single long one

4. Modify welded joints

5. Fillet Radius, Undercutting and Notch for Member in Bending

6. Drilling Additional Holes for Shaft

4. What are the advantages of welded joints compared with riveted joints?

## Advantages:

- The welded structures are usually light in weight compared to riveted structures. This is due to the reason, that in welding, gussets or other connecting components are not used.

- The welded joints provide high efficiency, which is not possible in the case of riveted joints.

- Alterations and additions can be made easily in the existing structures.

- Welded structures are smooth in appearance , therefore it looks pleasing.

- A welded joint has a great strength. Often a welded joint has the strength of the parent metal itself.

- It is easily possible to weld any part of a structure at any point. But riveting requires enough clearance.

- The process of making welding joints takes less time than the riveted joints.

- Shape like cylindrical steel pipes can be easily welded. But they are difficulty for riveting.

- The welding provides very strong joints. which can’t be bended easily. This is in line with the modern trend of providing rigid frames.

5. Write short note on Soderberg and Goodman lines

- Soderberg Line- A straight line joining Se on the ordinate to Syt on the abscissa is called the Soderberg line.

- Goodman Line- A straight line joining Se on the ordinate to Sut on the abscissa is called the Goodman line.

6. Explain Caulking and Fullering process in riveted joint with neat sketches.

Caulking is a process employed on riveted joints of pressure vessels like tanks, steam boiler to make joint leak proof and fluid tight. The narrow blunt chisel tool used for caulking is known as caulking tool.  Caulking is done at the edges of plates in lap joint and edges of strap plate in a butt joint.

Fullering is also a process to make a sealed joint. It is similar to the calking except the shape of tool. The tools used for this purpose is known as fullering tool. The thickness of fullering tool is equal to the thickness of plates. The fullering is more satisfactory than caulking because it gives a clean finish and less risk of damaging of plates.

8. What is fluctuating stress? Draw a stress–time curve for fluctuating stress.

Fluctuating stresses are those whose magnitude is varying with time. It is not necessary that the curve showing fluctuating stress passes through zero or mean stress by zero. Commonly the fluctuating stresses are classified as alternating stresses, reversed and repeated stresses. These stresses vary between their maximum (σmax) and minimum (σmin) values as shown in the Figure

9. Derive the condition for the transmission of maximum power of a belt drive system.

10. A wall bracket with a rectangular cross-section is shown in Figure. The depth of the cross-section is twice of the width. The force P acting on the bracket at 60° to the vertical is 5 kN. The material of the bracket is grey cast iron with ultimate tensile strength 200 MPa and the factor of safety is 3.5. Determine the dimensions of the cross-section of the bracket. Assume maximum normal stress theory of failure.